# EM950 Sensor Firmware Build Notes

| Product Name / Model: | EM950.XXXXX |
| --- | --- |
| Version: | Application v1.04, Boot v1.00 |
| Date: | September 21, 2023 |
| Project No.: | RES0800 |
| FW/SW Part No: | Firmware: 9074979, Programmed Part: 9074900, Ckt Bd: 9065600 |
| Technical Lead: | # Kratz, Fabian |
| Target Processor: (Hach p/n, Mfg p/n) | Mfg: STM32F103CBT6, Hach: 9064900 |

Purpose:

The purpose of this document is to instruct a software engineer on how to rebuild the EM950 firmware.

# Reference Documents

The items in the following table are reference documents that may provide guidance on building the product:

| Document Type | Document Citation |
| --- | --- |

# Build Environments

The items in the following table describe the environment(s) required to build the product:

| Environment | Name | Version | License Details | Other |
| --- | --- | --- | --- | --- |
| Operating System | Windows 7 Professional | SP1 |  |  |

# Build Tools

The items in the following table describe the environment(s) required to build the product:

| Tool | Name | Version | License Details | Other |
| --- | --- | --- | --- | --- |
| Toolchain | MDK-ARM Essential | V5.29.0.0 | LIC=SE754-FMSAA-80RNK-XKPBW-CZLPV-2AW1Q | OTT Hydromet GmbH |
| C Compiler | Armcc.Exe | V5.06 update 6 (build 750) | Installed and licensed with Realview |  |
| Assembler | Armasm.Exe | V5.06 update 6 (build 750) | Installed and licensed with Realview |  |
| Linker/Locator | ArmLink.Exe | V5.06 update 6 (build 750) | Installed and licensed with Realview |  |
| Librarian | ArmAr.Exe | V5.06 update 6 (build 750) | Installed and licensed with Realview |  |
| Hex Converter | FromElf.Exe | V5.06 update 6 (build 750) | Installed and licensed with Realview |  |
| CPU DLL | SARMCM3.DLL | V5.29.0.0 | Installed and licensed with Realview |  |
| Dialog DLL | DARMSTM.DLL | V1.66.0.0 | Installed and licensed with Realview |  |
| Target DLL | Segger\JL2CM3.dll | V2.99.36.0 | Installed and licensed with Realview |  |
| Dialog DLL | TARMSTM.DLL | V1.66.0.0 | Installed and licensed with Realview |  |
| Segger Debugger | Flasher Arm | Hardware | s/n: 174505635 |  |
| Segger Flash Utility | J-Flash Arm | V7.84d | NA |  |

# Build Input

The subsections below give the location of source code and other dependencies required to build the product.

## Location of Source Code

| Source Code | Location |
| --- | --- |
| FH950 Sensor | https://stash.waterqualitytools.com/scm/fh950/fh950_sensor.git |

## Location of Dependencies

| Dependency | Location |
| --- | --- |
| Finalize Build (“master” – Brunch) | https://stash.waterqualitytools.com/scm/fh950/fh950_sensor.git |

# Build Output

Using example working directory C:\Projects\FH950_Sensor\, the output resulting from building the product is located:

Application Firmware only

“C:\Projects\FH950_Sensor\Source\AppFlash\Output\ PVM_SENSOR_0_132_PXX_AC_VX_XX.bin”

Both Application and Boot Firmware only.

“C:\Projects\FH950_Sensor\Source\AppFlash\Output\ PVM_SENSOR_0_132_PXX_COMBINED_VX_XX.bin”

# Tool Setup and Installation

The following sub-sections provide step-by-step instructions for building each tool required to build the product.

## Keil uVision

- Install the KEIL uVision IDE using CD or the back up located at “http://fsbu_svn/svn/Software%20Tools/Trunk/Keil/”

- Installation of KEIL uVision is simple and is self explanatory, recommended to install on default path.

# Build Instruction

The following provides step-by-step instructions for building the product.

- Download the code from the subversion located at the

https://stash.waterqualitytools.com/scm/fh950/fh950_sensor.git

to the local computer at the path “C:\Projects\FH950_Sensor\Source”

- Open the KEIL uVision IDE

- Go to  Project=>Open Project” Browse through the above mentioned path at the local computer and open the project file “FH950_Sensor.uvmpw”

- Go to Project=>Rebuild (this process rebuilds all the files and the output is ready at the designed folder”)